TẠP CHÍ

KHOA HỌC

TRƯỜNG ĐẠI HỌC QUY NHƠN

# Thiết kế bộ quan sát H∞ theo phương pháp chia lưới cho hệ Lorenz 63 sử dụng mô hình dạng phi tuyến với tham số thay đổi

Võ Khánh Thoại1,*, Phạm Thanh Phong1,  Lê Tiến Dũng2,*

1Khoa Điện – Điện tử, Trường Đại học Sư phạm kỹ thuật, Đại học Đà Nẵng, Việt Nam

2Trường Đại học Bách Khoa, Đại học Đà Nẵng, Việt Nam

Ngày nhận bài: …/…/2025; Ngày sửa bài: …/…/2025; Ngày nhận đăng: …/…/2025; Ngày xuất bản: …/…/2025

## TÓM TẮT

Bài báo này đề xuất một phương pháp thiết kế bộ quan sát H∞  bền vững cho hệ Lorenz 63 thông qua mô hình hóa lại theo dạng hệ phi tuyến với tham số thay đổi (NLPV). Bằng cách sử dụng phương pháp chia lưới theo không gian trạng thái và xây dựng các mô hình tuyến tính cục bộ, chúng tôi thiết lập một tập hợp các bất đẳng thức ma trận tuyến tính (LMI) để thiết kế bộ quan sát tại mỗi điểm lưới. Các ma trận bộ quan sát thu được sẽ được nội suy theo thời gian thực dựa trên trạng thái của bộ quan sát. Phương pháp này cho phép ước lượng chính xác các trạng thái của hệ trong cả hai trường hợp có nhiễu và không nhiễu. Kết quả mô phỏng và so sánh với bộ lọc Kalman mở rộng (EKF) xác nhận hiệu quả của phương pháp đề xuất thông qua các chỉ số đánh giá RMSE, NRMSE và hệ số tương quan R2

Từ khóa: Hệ thống Lorenz, Bộ quan sát H∞ , Hệ phi tuyến với tham số thay đổi (NLPV), Tiếp cận LMI, Phương pháp chia lưới

*Tác giả liên hệ chính.

Email: vkthoai@ute.udn.vn, ltdung@dut.udn.vn

https://doi.org/10.52111/qnjs.2025.xxxxx

Tạp chí Khoa học Trường Đại học Quy Nhơn, 2025, xx(x), 137-148  xxx

# Gridding-Based H∞ Observer Design for the

# Lorenz 63 System Using an NLPV Reformulation

Khanh-Thoai Vo1,*, Thanh-Phong Pham1, Tien Dung Le2,*

1University of Technology and Education (UTE), The University of Danang (UD),

2University of Science and Technology (DUT), The University of Danang (UD),

Received: xx/xx/2025; Revised: xx/xx/2025;

Accepted: xx/xx/2025; Published: xx/xx/2025

## ABSTRACT

This paper presents a robust H∞ observer design for the Lorenz 63 system based on a Nonlinear Parameter-Varying (NLPV) reformulation. The nonlinear dynamics are approximated by gridding the state space and constructing local linear models. At each grid point, an observer gain is synthesized by solving a linear matrix inequality (LMI), with a common Lyapunov function ensuring stability across the operating range. The observer gain is updated online through barycentric interpolation based on the current estimated state. The approach enables real-time, robust state estimation in the presence of model nonlinearities and disturbances. Simulation results under both noisy and noise-free conditions and comparison with an Extended Kalman Filter (EKF) confirm the effectiveness of the proposed design. Quantitative evaluations using RMSE, NRMSE, and R2 demonstrate high estimation accuracy and robustness of the observer across a range of dynamic behaviors in the Lorenz 63 system.

Keywords: Lorenz system, H∞ observer, Nonlinear parameter-varying (NLPV), LMI approach, Gridding method.

## INTRODUCTION

The Lorenz 63 system is a classical benchmark in nonlinear dynamics and chaos theory.1-3 Due to its strong nonlinearities and sensitivity to initial conditions, it provides an ideal platform for testing observer design techniques. Designing observers for such systems is challenging, especially when dealing with unknown inputs, external disturbances, and nonlinearities.4-8

Traditional methods like the EKF often rely on linearization and statistical  assumptions,  which may not provide robustness in chaotic regimes.9-10

An alternative is the 𝐻∞ observer framework, which focuses on worst-case disturbance attenuation.11-13 However, applying 𝐻∞ methods directly to nonlinear systems is difficult due to non-convexity.1

To overcome this issue, the nonlinear system can be reformulated into a NLPV structure. The state space is discretized through gridding, and LMIs are employed to design observer gains at multiple linearization points.  During online execution, the observer gain is interpolated in real-time based on the current estimated state using barycentric weights.14

*Corresponding author.

Email: vkthoai@ute.udn.vn, ltdung@dut.udn.vn

The proposed approach is validated using the Lorenz 63 system. Estimation performance is evaluated under both noisy and noise-free scenarios using standard quantitative metrics, including Root Mean Square Error (RMSE), Normalized RMSE (NRMSE), and the coefficient of determination ( R2). Results confirm the robustness and effectiveness of the observer design across a wide range of operating conditions.

The main contributions of this paper are summarized as follows:

- A deterministic NLPV reformulation of the stochastic Lorenz 63 system using expectation and Itô correction.

- A grid-based H∞ observer synthesis procedure using LMIs and a common Lyapunov function.

- A real-time gain scheduling strategy using barycentric interpolation based on the observer state.

- Simulation-based performance evaluation under noisy and noise-free conditions using RMSE, NRMSE, and R2  metrics.

The rest of this paper is organized as follows. Section II presents the NLPV modeling of the Lorenz system. Section III introduces the observer design approach via gridding and LMIs. Section IV provides numerical simulations and evaluation. Section V concludes the paper.

## MODELLING AND NLPV REFORMULATION

This section presents a systematic reformulation of the original second-order Stochastic Differential Equation (SDE) into a deterministic NLPV model. This transformation facilitates tractable observer synthesis using convex optimization techniques, such as LMI.

### Original Stochastic Model

### The system under consideration is governed by a nonlinear SDE of the form9:

dyt=fytdt+gytq12tdβt+MhyytR-1(t)

dzt+R1/2(t)dut-2hytdt	(1)

where:

- yt∈Rn denotes the state vector,

- f(⋅) is the nonlinear drift function,

- g(⋅)q1/2(t) represents the process diffusion term,

- h(⋅) is a nonlinear measurement function,

- M(t)∈Rn×n is the estimation covariance matrix,

- R(t)∈Rp×p is the measurement noise covariance matrix,

- βt,ut are standard Wiener processes.

The last term on the right-hand side resembles the innovation update in ensemble-based filters and includes both measurement information and gain-based corrections.

### Deterministic Approximation via Expectation

To obtain a tractable deterministic model, the expectation of both sides of the SDE is considered. Since the expected values of Wiener process increments are zero, the stochastic terms vanish, yielding9,15:

dytdt=Efyt+12Mfyyyt+MhyytR-1t

zt-hyt-12Mhyyyt	(2)

Here, yt=Eyt denotes the expected state trajectory, and fyy,hyy denote the second-order derivatives (Hessians) of f and h, respectively. The term involving 12Mfyy results from Itô's correction.16

### NLPV Model Formulation

Let x(t):=yt represent the observer's estimated state. Using first-order Taylor expansion, the nonlinear function f(x) and hx are approximated by their Jacobians and higher-order residual terms. The system dynamics can then be written as:

x(t)=A(ρ(t))x(t)+B(ρ(t))u(t)+frem (x(t))	(3)

where:

- A(ρ)=∇f(x)+12Mfyy(x) : linearized drift matrix with second-order correction,

- Bρ=MhyxR-1(t): input matrix associated with measurement innovation,

- frem (x(t)): residual nonlinearities arising from approximation errors,

- ρt≔xt,Mt,Rt: scheduling parameter vector.

The remainder term frem (x(t)):  captures the higher-order nonlinear dynamics not explicitly modeled in A(ρ) or Bρ, and is assumed to satisfy a Lipschitz condition:

frem x1-frem (x2)≤Lfx1-x2 ∀x1,x2	(4)

## OBSERVER DESIGN VIA GRIDDING

This section presents the design of a robust state observer for NLPV systems affected by both process disturbances and nonlinear uncertainties. The observer is synthesized using gridding techniques and LMI conditions, ensuring an H∞ performance level.

### System Model with Nonlinearity and Disturbance

Consider the nonlinear system with parameter-varying structure:

xt=Aρtxt+Bρtut+Bwρtwt+fremxt;

y(t)=Cx(t)+Dw(ρ(t))w(t)	(5)

where x(t)∈Rn is the system state, u(t)∈Rm is the control input, w(t)∈Rnw is the exogenous disturbance, and frem (x) is a nonlinear remainder function satisfying a Lipschitz condition in (4).

### Observer Structure

The structural form of the observer is presented as follows:17

xt=Aρtxt+Bρtut+frem xt+Lρtyt-yt;	(6)

y(t)=Cx(t)

Let e(t)=x(t)-x(t) denote the estimation  error

error. The error dynamics are derived as:

et=Aρt-LρtCet+Δf(t)

+Bwρt-L(ρ(t))Dw(ρ(t))w(t) 	(7)

where Δf(t)=frem (x(t))-frem (x(t)) and satisfies ‖Δf(t)‖≤Lf‖e(t)‖.

### LMI Condition for H∞ Observer

To ensure robustness against w(t) and nonlinear uncertainties, a quadratic Lyapunov function V(e)= e⊤Pe is considered, with P>0. The following condition is imposed:

V(e)+z⊤z-γ2w⊤w≤0, z=Ce+Dww 	(8)

Calculate the derivative:

V(e)=e⊤(A-LC)⊤Pe+e⊤P(A-LC)e

+ 2e⊤PBw-LDww+2e⊤PΔf 	(9)

Use the inequality18,19:

2e⊤PΔf≤ϵf‖Δf‖2+1ϵfe⊤P2e

≤ϵfLf2‖e‖2+1ϵfe⊤P2e	(10)

Combined:

V(e)≤e⊤(A-LC)⊤P+P(A-LC)+ϵfLf2Ie

+ 2e⊤PBw-LDww+1ϵfe⊤P2e	(11)

Create concatenation vector:

ξ=ew,  z=Ce+Dww	(12)

Apply the matrix inequality:

V+z⊤z-γ2w⊤w≤ξ⊤Ψξ<0	(13)

Return to Standard LMI Form at each gridding vertex:

Y=-PL⇒L=-P-1Y 	(14)

Ai=Aρi,Bw,i=Bwρi,Dw,i=Dwρi 	(15)

Applying standard manipulations and bounding Δf(t) via the Lipschitz property, the inequality

reduces to the feasibility of the following LMI at each grid point ρi:

Ξi+ϵfLf⊤LfΦiC⊤Φi⊤-γ2IDw,i⊤CDw,i-I<0	(16)

with definitions:

Ξi=AiP+PAi⊤+C⊤Yi⊤+YiC	(17)

Φi=PBw,i+YiDw,i	(18)

To ensure exponential convergence of the estimation error, a decay rate condition is embedded within the Lyapunov framework. Specifically, for a candidate Lyapunov function 𝑉(𝑒) =e⊤Pe, where 𝑃 > 0, the derivative along the estimation error dynamics is required to satisfy:

V(e)≤-2βV(e) 	(19)

This implies:

e⊤(Ai⊤P+PAi)e≤-2βe⊤Pe	(20)

which leads to the matrix inequality:

Ai⊤P+PAi+2βP≤0	(21)

Here, the scalar β > 0 denotes a desired minimum decay rate of the estimation error dynamics, directly controlling the convergence speed of the observer. The term 2βP is therefore incorporated into the LMI formulation to guarantee that the estimation error decays at an exponential rate of at least β. This ensures a desired performance level in transient response.

Accordingly, the main LMI condition at each grid point ρi: includes this term as follows:

AiP+PAi⊤+C⊤Yi⊤+YiC+ 2βP+ϵfLf⊤Lf<0 				(22)

The inclusion of 2βP acts analogously to pole placement in linear observer design, where 2β controls the speed of the eigenvalues of the error dynamics. This approach enables the systematic shaping of observer convergence through convex optimization. And LMI in (16) is

rewritten:

Ξi+ 2βP+ϵfLf⊤LfΦiC⊤Φi⊤-γ2IDw,i⊤CDw,i-I<0	(23)

If feasible, the observer gains at the grid point ρi: is recovered as Li=-P-1Yi

### Gridding and Barycentric Interpolation Approach

To effectively design an observer for nonlinear parameter-varying systems, the state-dependent matrices Aρt, Bρtand Lρt must be approximated. Direct continuous-time synthesis is generally intractable due to infinite-dimensional dependency on the scheduling parameter ρt. Therefore, a gridding approach is adopted to discretize the state space.

The state space X ⊂ x(t)∈Rn is partitioned into a finite number of grid points {x[i]}1=1N​, where a local LMI observer synthesis is performed. At each grid point x[i], an observer gain L[i] is computed by solving the corresponding LMI condition. These gains are stored for online use.

To enable smooth gain variation and avoid chattering between discrete observers, the gains are interpolated during runtime using barycentric weights. Let x(t) denote the current observer state. The interpolated gain L(xt) is calculated as:

Lxt=i=1Nixt L[i],	(24)

where i(⋅) are barycentric interpolation weights satisfying  i=1Nixt =1, and i≥0.

## NUMERICAL SIMULATION

This section validates the proposed gridding-based H∞ observer on the chaotic Lorenz 63 system under both noisy and noise-free conditions. Performance is evaluated using standard error metrics.

### System Setup

The Lorenz 63 system is a well-known nonlinear chaotic system governed by the following differential equations:

xt=Bwwt+fxt,y(t)=Cx(t)+Dw w(t)	(25)

where x(t)=[x1, x2,x3] ⊤∈R3is the system state, w(t)∈R3denotes external disturbances, and the measurement output is yt∈R.The matrices are defined as:

yt=Cxt,  C=100.	(26)

The nonlinear vector field f(x) is given by1:

fx(t)=σx2-x1x1ρ-x3-x2x1x2-βx3 	(27)

with parameters σ=10,ρ=28, β=83. To facilitate observer design using convex optimization tools, we reformulate the nonlinear system into an NLPV structure by approximating the dynamics through local linearizations.

The nonlinear vector field f(x) is linearized around multiple grid points x(i)i=1N within a bounded region D⊂R3. At each point x(i), the Jacobian matrix is computed as:

A(i):=Jx(i)=∂f∂xx(i)	(28)

This results in a set of locally linearized models:

x(t)≈A(i)x(t)+Bww(t)+frem(x)	(29)

where frem (x):=f(x)-A(i)x is the residual nonlinearity. Assuming that f(x) is Lipschitz continuous over D, the residual satisfies:

frem (x)-frem (x)≤Lf‖x-x‖, ∀x,x∈D	(30)

for some constant Lf>0. To express the system in NLPV form, we introduce a parameter trajectory ρ(t)=x(t), leading to:

x(t)=A(ρ(t))x(t)+Bww(t)+frem (x)y(t)=Cx(t)+Dww(t)	(31)

The matrix A(ρ(t)) is obtained via online interpolation of A(i) using barycentric weights:

Aρt=i=1N iρtAi, 	(32)

where i i=1, i≥0

This interpolation ensures a smooth and accurate approximation of the nonlinear dynamics across the grid.

### Lipschitz Constant Estimation

The observer gains are designed at grid points uniformly sampled over [-20, 20]3. At each grid point, the local Jacobian is computed and used to define Aρi.

The nonlinear drift term f(x) of the Lorenz 63 system is defined in equation (27). To facilitate observer design with Lipschitz-type nonlinearities, the Lipschitz constant Lf is required. A vector field f(x) is said to be Lipschitz continuous over domain D⊂Rn if there exists a scalar Lf>0 which satisfies the equation (30).

A sufficient condition to obtain Lf is to evaluate the spectral norm of the Jacobian matrix J(x):

Lf=supx∈D ‖J(x)‖2	(33)

where Jx=∂fx∂x  is the Jacobian matrix of the drift function.

For the Lorenz system, the Jacobian is computed as:

J(x)=∇f(x)=-σσ0ρ-x3-1-x1x2x1-β	(34)

The spectral norm ‖J(x)‖2 is the largest singular value of J(x), which can be numerically evaluated over a bounded region D. In this work,

the domain is chosen as D=[-20, 20]3, covering the typical range of Lorenz state trajectories.

A grid-based scan of J(x) across D yields an upper bound: Lf≈56.6092

Fig. 1 describes the distribution of ‖J(x)‖2​ over a bounded domain in (x1, x2,x3). It is observed that the spectral norm varies significantly, reaching values above Lf=56.6092 in lower regions of the state space (i.e., x3< 0), while staying below 25 in upper regions (i.e., x3 >10). This spatial variability reflects the strong local nonlinearity of the Lorenz system, which motivates the use of a gridding-based observer design. By selecting local Lipschitz bounds within each grid cell, the observer gain can be adapted more accurately to the system's local dynamics, avoiding conservatism associated with a global Lipschitz constant.

This bound is used in the LMI formulation to handle the nonlinear remainder term using Lipschitz inequalities, ensuring robust estimation even in the presence of nonlinear uncertainties.

Using CVX toolbox and semidefinite programming (SDP)20, a set of LMI conditions is solved to obtain observer gains Li. At runtime, the gain L(x) is interpolated using barycentric weights based on proximity to the grid centers.

Figure 1.  A graph representation of the Jacobian ‖Jx‖2 in the state space.

Specifically, for each grid point 𝑖 ∈{1, 2,…, 4} of the NLPV system in equation (24), the LMI condition in equation (23) is solved to synthesize a robust H∞ observer.

The CVX toolbox in MATLAB is employed to solve the optimization problem and compute the observer gain matrices Li  corresponding to each value of the scheduling parameter ρ, using the system matrices defined in section 3.4.

The optimization yields a disturbance attenuation level of γ∞= 0.0023. The resulting observer gain L(x)​ of the four grid points of ρ are as follows:

L(x) =[519.9294; -17.1069; 6.3601]T

### Simulation Scenarios and Discussion

To evaluate and compare the performance of the proposed H∞ observer and the EKF9, numerical simulations are carried out on the Lorenz 63 chaotic system. The initial state of the system is set as x(0)=[-5,-5,-5]T, and both observers are initialized at the origin [0, 0, 0]T. The EKF is implemented using a first-order prediction-correction structure, where the time-varying Jacobian matrix of the Lorenz system is used in the prediction step. The initial covariance matrix is selected as P0= 2⋅I3 to ensure sufficient initial uncertainty for the EKF. In contrast, the H∞ observer uses a gridding structure interpolated via barycentric weights over a predefined grid of the state space.

To provide a comprehensive performance analysis, two sets of simulation conditions are examined:

- Noise-Free Scenario: The system evolves without any disturbances to establish a baseline.

Figure 2 illustrates the comparison between the actual states x1 to x3  and their corresponding estimated values x1 to x3. In the plots, the solid green line represents the true system states, the red dashed line indicates the estimates from the proposed H∞ observer, while the blue dash-dot line corresponds to the estimates obtained using the EKF.

Figure 2. Comparison of system states with noise-free

It is evident from the Fig. 2 plots that both H∞ observer and the EKF closely track the true system states. The estimation errors are negligible for all state variables, reflecting excellent accuracy in the absence of noise. Notably:

In x1, both observers almost overlap with the ground truth across the entire time span, with only very slight divergence in highly dynamic segments. The inset zoomed plots confirm sub-millisecond response agreement.

In x2, the estimations remain aligned even during sharp transient oscillations. This highlights the observers’ ability to capture rapid nonlinear dynamics.

In x3, where chaotic oscillations dominate, both xH∞ and xEKF  accurately replicate the system evolution.

The detailed zoom-in windows emphasize that the grid-based H∞​ observer performs on par with EKF under ideal conditions, while offering the added benefit of robustness in the presence of modeling uncertainties, which is discussed further in the noisy scenarios.

- Noisy Scenario: Both process and measurement noise are activated.

The state dynamics are subjected to an additive zero-mean Gaussian process noise w(t)∼N(0,Q) and measurement noise v(t)∼ N(0,R), where the covariance matrices are chosen as follows:

Q=0.5⋅I3, R=I1	(35)

The process noise w(t) is generated at each time step as:

wk=Q⋅ωk, ωk∼N0,I3	(36)

while the measurement noise is:

vk=R⋅νk, νk∼N0,I1	(37)

Figure 3 compares the actual states x1 to x3 and their corresponding estimated values x1 to x3, in the presence of noise as described above. The figures demonstrate that both observers track the system well, but the performance diverges during fast transients and in regions of strong nonlinear coupling.

During sharp state transitions (e.g., 𝑡 ≈ 5.2s and 𝑡 ≈ 17.7s), the EKF estimator shows noticeable deviations from the ground truth, especially in x1  and x2 (see zoomed-in insets). This is attributed to the EKF’s reliance on local linearization, which becomes inaccurate under high system curvature.

The proposed H∞ ​ observer, designed via gridding and interpolation of locally optimized observer gains Li, exhibits uniform tracking accuracy even under noise. This robustness stems from its design via LMI constraints incorporating Lipschitz bounds, which explicitly account for nonlinear uncertainty in the estimation error dynamics.

Figure 3. Comparison of system states with noise

Tables 1–3 present the state estimation performance indices - including  Root Mean Square Error (RMSE), Normalized RMSE (NRMSE) and the Coefficient of Determination  (R2) - for both the EKF and the proposed 𝐻∞ observer, under noise-free and noisy conditions, respectively.

- Root Mean Square Error (RMSE):

RMSEi=1Nk=1N  xi(k)-xi(k)2	(38)

- Normalized RMSE (NRMSE):

NRMSEi=RMSEimaxxi-minxi	(39)

- Coefficient of Determination R2 :

Ri2=1- xi-xi2 xi-xi2	(40)

Table 1. RMSE index of states

| RMSE | With noise-free | With Noise |
| --- | --- | --- |
|  | EKF H∞ | EKF H∞ |
| x1 | 0.04042 0.00197 | 0.25921 0.00313 |
| x2 | 0.06177 0.15654 | 0.33442 0.16664 |
| x3 | 0.07490 0.16884 | 0.27328 0.17989 |

Table 1. NRMSE index of states

| N RMSE | With noise-free | With Noise |
| --- | --- | --- |
|  | EKF H∞ | EKF H∞ |
| x1 | 0.00114 5.62e-05 | 0.00757 9.14e-05 |
| x2 | 0.00131 0.00336 | 0.00743 0.00370 |
| x3 | 0.00197 0.00455 | 0.00679 0.00447 |

Table 3. 𝑅2 index of states

| 𝑅2 | With noise-free | With Noise |
| --- | --- | --- |
|  | EKF H∞ | EKF H∞ |
| x1 | 0.99996 1.00000 | 0.99896 0.99999 |
| x2 | 0.99995 0.99968 | 0.99857 0.99964 |
| x3 | 0.99991 0.99955 | 0.99872 0.99944 |

The simulation results demonstrate a comprehensive comparison between the proposed H∞ observer and the EKF under both noise-free and noisy conditions. As presented in Tables 1–3, the H∞ The observer exhibits superior robustness, especially in the presence of process and measurement noise.

In the noise-free scenario, EKF achieves slightly better RMSE values for x2​ and x3​, while the H∞ observer delivers the best accuracy for x1​, achieving an RMSE of only 0.00197 and a coefficient of determination R2 = 1.0000. However, in the noisy case, the performance of EKF significantly degrades across all states. For instance, the RMSE for x1​ increases to 0.25921 under EKF, whereas the proposed observer maintains a remarkably low RMSE of 0.00313.

The NRMSE analysis further supports these findings, with the H∞ observer consistently achieves lower normalized errors under noisy conditions. Specifically, for x1​, the NRMSE of the H∞ observer remains as low as 9.14×10−5, compared to 7.57×10−3 for EKF.

In terms of the coefficient of determination, the H∞ observer consistently attains higher R2 values in both scenarios, indicating a better match between estimated and actual states. Notably, the observer preserves an R2 of over 0.9999 for all states even in the presence of noise, whereas EKF drops to 0.9985 or lower.

Overall, these results validate the robustness and estimation accuracy of the proposed H∞ observer design. The gridding-based LMI synthesis, combined with barycentric interpolation of the observer gain, enables the observer to maintain high precision under strong nonlinearities and measurement uncertainties. In contrast, EKF performance is more sensitive to noise and model mismatch, highlighting the conservative yet effective design philosophy of the H∞ approach.

## CONCLUSIONS

This paper presented an H∞ observer design for the Lorenz 63 chaotic system using a NLPV framework combined with a Lipschitz-based approach. By leveraging gridding techniques and convex optimization via semidefinite programming, the observer gains were synthesized at predefined grid points and interpolated in real time based on barycentric weights.

Simulation results demonstrated that the proposed H∞ observer provides superior estimation accuracy and robustness compared to the EKF, particularly under process noise conditions. Quantitative metrics such as RMSE, NRMSE, and R2 confirmed the consistent performance improvements of the proposed method.

This framework offers a practical and scalable solution for state estimation in nonlinear systems with strong local dynamics. Future work will explore extensions to output-feedback control, observer-based synchronization of chaotic systems, and application to more complex NLPV systems such as robotic manipulators or fluid dynamics.

## REFERENCES

- E. N. Lorenz. Deterministic Nonperiodic Flow,” J. Atmos. Sci., 1963, 20(2),  130–141.

- M. Goodliff, S. Fletcher, A. Kliewer, J. Forsythe, A. Jones.  Detection of non-Gaussian behavior using machine learning techniques: a case study on the Lorenz 63 model, Journal of Geophysical Research: Atmospheres, 2020, 125(2), e2019JD031551.

- P. Tandeo, P. Ailliot, J. Ruiz, A. Hannart, B. Chapron, A. Cuzol, and R. Fablet. Combining analog method and ensemble data assimilation: application to the Lorenz-63 chaotic system, Proceedings of the 4th International Workshop on Climate Informatics , Springer International Publishing, 2015, 3-12.

- F. Zhu, Y. Fu, T. N. Dinh. Asymptotic convergence unknown input observer design via interval observer, Automatica, 2023, 147, 110744. 10.1016/j.automatica.2022.110744

- C. M. Nguyen, A-T. Nguyen, S. Delprat.  Cascade Takagi-Sugeno Fuzzy Observer Design for Nonlinear Uncertain Systems with Unknown Inputs: A Sliding Mode Approach, International Journal of Robust and Nonlinear Control, 2023, 33(15) 9066-9083 DOI: 10.1002/rnc.6371

- T-P. Pham, O. Sename, L. Dugard. A nonlinear parameter varying observer for real‐time damper force estimation of an automotive electro‐rheological suspension system, International Journal of Robust and Nonlinear Control, 2021, 31(17),8183-8205. https://doi.org/10.1002/rnc.5583

- S. Meng, F. Meng, F. Zhang, Q. Li, Y. Zhang, A. Zemouche. Observer design method for nonlinear generalized systems with nonlinear algebraic constraints with applications, Automatica, 2024, 162, 111512.  10.1016/j.automatica.2024.111512

- S. Mohite, M. Alma, A. Zemouche. Toward enhancing nonlinear observers for Lipschitz system: Exploiting the matrix multipliers‐based LMIs, International Journal of Robust and Nonlinear Control, 2024, 34(15), 10799-10820. https://doi.org/10.1002/rnc.7552

- C. O. Ongere, D. Angwenyi, R. Oryiema. Second order Extended Ensemble Kalman Filter with Stochastically Perturbed Innovation, Science Mundi, 2024, 4(2), 222-242

- Hazazi, Muhammad Asaduddin and Sihabuddin. Extended Kalman filter in recurrent neural network: USDIDR forecasting case study, Indonesian Journal of Computing and Cybernetics Systems, 2019, 13(3), 293. DOI:10.22146/ijccs.47802

- S. Mohite, M. Alma, A. Zemouche, M. Haddad. LMI-based H∞ observer design for nonlinear Lipschitz system, IFAC-PapersOnLine,  2023, 56 (2), 6745-6750.10.1016/j.ifacol.2023.10.380

- E. Chnib, P. Bagnerini, A. Zemouche. LMI based H∞ Observer Design for a Quadcopter Model Operating in an Adaptive Vertical Farm, IFAC-PapersOnLine, 2023, 56(2), 10837-10842. https://doi.org/10.1016/j.ifacol.2023.10.757

- K-T. Vo, T. D. Le, T-P. Pham. Design of an H∞ observer for nonlinear parameter-varying systems with lipschitz nonlinearity and model uncertainty, International Conference on Technology Innovation for Sustainable Development (TI4SD) 2025.

- J-P. Berrut, L. N. Trefethen. Barycentric Lagrange Interpolation, Society for Industrial and Applied Mathematics, 2004, 46(3), 501–517.  DOI. 10.1137/S0036144502417715

- A. H. Jazwinski. Stochastic Processes and Filtering Theory, 1970, iii-ix, 1-376

- K. Ito. 109. Stochastic Integral, Proceedings of the Imperial Academy, 1944, 20(8), 519-524

- D. G. Luenberger. Observing the state of a linear system”, IEEE Transactions on military electronics, 1964,  8 (2), 74-80.

- H. K. Khalil, Nonlinear Systems, 3rd Ed., Prentice Hall, 2002.

- S. Boyd and L. El Ghaoui et al., Linear Matrix Inequalities in System and Control Theory, SIAM, 1994.

- M. Grant, and S. Boyd (2014). CVX: Matlab software for disciplined convex programming, version 2.1. http://cvxr.com/cvx